DRAFT FOR REVIEW

# Flight Test Configuration Readiness Checklists

One checklist for each type of Configuration Item (CI) installed on a flight test aircraft

## Purpose

A CI flies in a known configuration. A known configuration is a baseline, plus its approved changes, plus any variances on that unit.

Flight test units are usually built to the developmental configuration. Most fly on the initial product baseline, before any physical configuration audit. MIL-HDBK-61B notes that units built in this phase “are not necessarily representative of the production hardware.” So each CI needs its own proof of a known configuration before first flight. These checklists collect that proof.

The checklists derive from the company CI identification process, the configuration baselines guidance, EIA-649-1A, GEIA-HB-649A, and MIL-HDBK-61B. They add no requirements. Where the contract or the process differs, the contract and the process govern.

## Four questions

Each checklist groups its artifacts under four questions.

| Definition | What must the CI be? The requirements, interfaces, analyses, verifications, and design at revision. |
| --- | --- |
| Evidence | Why is the CI cleared to fly? Qualification results, the approval and clearance notice, and the difference between the qualified unit and the flight unit. |
| The unit | What is actually on this tail? The as-built record, acceptance results, variances, changes, and installation. |
| Status | Is the CM record clean? The CI index and the interface register. |

## How to use the checklists

- Complete one checklist for each CI installed on the test tail. Pick the checklist that matches the CI type in the table below.

- Check an item only when the evidence exists at the revision that matches the flight configuration.

- Record the document number, revision, and location of the evidence in the checklist.

- Mark an item N/A only with a written reason.

- An item that cannot close before flight goes on the consolidated limitations list, or the flight does not go.

- The system checklist closes last. It depends on every other checklist.

## Checklist index

| Prefix | CI type | Worked example | Who completes it |
| --- | --- | --- | --- |
| S | System (top-level CI) | Air vehicle | CM, with Program Management |
| A | Acquirer-spec controlled CI | Flight control system | CM, with Systems Engineering |
| H | Company-design hardware CI | Control electronics unit | CM, with Design Engineering |
| W | Software CI (CSCI) | Flight control software | CM, with Test and Verification |
| P | Supplier-design hardware CI on a source control drawing | Elevator actuator | CM, with Supplier Requirements Engineering |
| G | Government furnished equipment CI | Government-furnished radio | CM, with Program Management |
| N | Items that fly but are not CIs | Flight test instrumentation | CM, with Test and Verification |

## Terms

| Term | Definition | Source |
| --- | --- | --- |
| Approval and clearance notice (ACN) | The output of qualification testing conducted on a CI. It identifies the current exceptions and limitations associated with the CI. | Company usage |
| Development configuration | The contractor’s design and associated technical documentation that defines the contractor’s evolving design solution during development of a CI. | MIL-HDBK-61B 5.5.2 d |
| Current approved configuration | A baseline for any product, or any document, plus the approved changes, is the current approved configuration. | EIA-649-1A Principle CI-12 |
| Variance | An authorized temporary departure from the approved configuration documentation information for a specific number of units under the contract or contracts identified and that does not require revision of approved product configuration documentation. | EIA-649-1A 2.3 |
| As-built configuration | The configuration of an item as actually produced. The as-built configuration consists of the as-designed configuration at the time of production as modified with approved engineering changes and variances. | EIA-649-1A 2.3 |
| Tail configuration list | The record of every CI installed on one flight test aircraft, with its PIN, revision or software version, serial or lot number, ACN, and open limitations. | This checklist set |
| Consolidated limitations list | The record of every open ACN limitation and every RFV use restriction on one flight test aircraft. | This checklist set |

## Known open item

The process does not yet say how configuration status accounting records each unit’s ACN and its open limitations. Until it does, the tail configuration list and the consolidated limitations list are built by hand. Treat both as controlled records.

# Checklist S.  System (top-level CI)

Example: the air vehicle. Control tier: Acquirer-spec controlled.

The system checklist closes last. It depends on a completed checklist for every CI installed on the tail.

The system is the only CI that needs a flight clearance. That clearance names the whole tail configuration.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the system must be |  |  |  |  |  |  |
| ☐ S.1 | Contract and statement of work at the current modification. | The obligations the flight must meet. | Program Management | EIA-649-1A 3.2.3.2 note |  |  |
| ☐ S.2 | System specification at its approved revision (FBL). | What the system must do. | Systems Engineering | EIA-649-1A 3.2.3.2 (1) a |  |  |
| ☐ S.3 | External interface requirements, and the ICDs the system specification invokes, at revision. | External connections are defined. | Systems Engineering | EIA-649-1A 3.2.5.1 (2); MIL-HDBK-61B 5.5.2.6 |  |  |
| ☐ S.4 | Interface register: every row on the system mapped. Structural floor complete. No open coverage exceptions. | Every identified interface is controlled. | CM | EIA-649-1A 3.2.5; company CI identification process |  |  |
| ☐ S.5 | System safety hazard analysis reports, current to the flight configuration. | The hazard controls match the aircraft that will fly. | System Safety | EIA-649-1A 3.2.3.2 (1) a |  |  |
| ☐ S.6 | System verification plan at revision. | How each system requirement will be shown. | Test and Verification | EIA-649-1A 3.2.3.2 (1) b |  |  |
| ☐ S.7 | System EBOM and installation drawings for the tail, at revision, with engineering release records. | What the aircraft was built to. | Design Engineering, CM | EIA-649-1A 3.2.2; 3.2.3.2.2 (1) a |  |  |
| ☐ S.8 | Flight test instrumentation definition: CI screening result, installation drawings, and interface rows. See the checklist for items that fly but are not CIs. | Test equipment on the aircraft is defined and controlled. | Test and Verification, CM | Company CI identification process |  |  |
| Evidence: why the system is cleared to fly |  |  |  |  |  |  |
| ☐ S.9 | A completed checklist for every CI installed on the tail. | Each CI is cleared on its own. | CM | This checklist set |  |  |
| ☐ S.10 | Consolidated limitations list. It holds every open ACN limitation and every RFV use restriction on the tail. | The aircrew and the test team know what is not cleared. | Test and Verification, System Safety | MIL-HDBK-61B Table I |  |  |
| ☐ S.11 | Status of the test and certification requirements due before flight. Each one is closed or dispositioned. | Nothing required before flight is open. | Test and Verification | MIL-HDBK-61B Table I |  |  |
| ☐ S.12 | Flight clearance from the Acquirer’s airworthiness authority, naming the tail configuration. | The Acquirer authorizes this configuration to fly. | Program Management | GEIA-HB-649A FCA checklist (flight approvals) |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ S.13 | Tail configuration list. It names every installed CI with its PIN, revision or software version, serial or lot number, ACN, and open limitations. | What is actually on this aircraft. | CM | MIL-HDBK-61B Table I (as-built configuration) |  |  |
| ☐ S.14 | Physical verification of the tail against the tail configuration list. | The record matches the hardware. | Quality, CM | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ S.15 | Approved system-level RFVs, with their use restrictions. | Each known departure is acceptable for flight. | CM | EIA-649-1A 3.3.2; MIL-HDBK-61B 6.2 |  |  |
| ☐ S.16 | Change incorporation record and list of open ECPs against the tail. | The current approved configuration of the tail. | CM | Principle CI-12; EIA-649-1A 3.4.1 (3) |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ S.17 | CI index: every installed CI is designated, Acquirer concurred, and its baseline status is recorded. | Every CI on the tail is managed. | CM | Company CI identification process |  |  |
| ☐ S.18 | Configuration documentation package delivered for the test readiness review. | The review saw the real configuration. | CM | EIA-649-1A 3.1.3 (1) f |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist A.  Acquirer-spec controlled CI

Example: the flight control system. Its specification is under Acquirer control.

This CI sits at the top tier below the system. The Acquirer controls its specification once it approves the ABL.

Any Class I change or major RFV against this specification needs Government approval before flight.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the CI must be |  |  |  |  |  |  |
| ☐ A.1 | CI specification at its approved revision (ABL), traced to the system specification. | What the CI must do. | Systems Engineering | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ A.2 | Interface requirements, and the ICDs the specification invokes, at revision. | The CI’s connections are defined. | Systems Engineering | EIA-649-1A 3.2.5.1 (2); MIL-HDBK-61B 5.5.2.6 |  |  |
| ☐ A.3 | Interface register: every row on this CI mapped. Parent and child rows across control tiers present. No open coverage exceptions. | Every identified interface is controlled. | CM | Company CI identification process |  |  |
| ☐ A.4 | FMEA, FMECA, and hazard analyses for this CI, current to the flight configuration. Each allocated critical function is stated in the specification. | The analyses describe the hardware that will fly. | System Safety, RM&SH | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ A.5 | Verification plan, qualification and integration test procedures, and qualification analyses, at revision. | How the CI is shown to meet its specification. | Test and Verification | EIA-649-1A 3.2.3.2.1 (1) b |  |  |
| ☐ A.6 | Installation drawings at revision, with engineering release records. List of child CI product baselines. | What the CI was built and installed to. | Design Engineering | EIA-649-1A 3.2.2; 3.2.3.2.2 (1) a |  |  |
| Evidence: why the CI is cleared to fly |  |  |  |  |  |  |
| ☐ A.7 | Qualification and integration test results, tied to the exact configuration tested, including the lab or rig configuration. | The evidence applies to this design. | Test and Verification | MIL-HDBK-61B App C (correlate prototype configurations to test results) |  |  |
| ☐ A.8 | ACN naming the cleared CI configuration, with its current exceptions and limitations. | What the CI may and may not do in flight. | Test and Verification | Company usage; EIA-649-1A 3.3.1.2 (2) w |  |  |
| ☐ A.9 | Qualification-to-flight difference assessment. | The flight configuration is covered by the ACN. | Test and Verification, Systems Engineering | EIA-649-1A 3.5.3.1 (1) b, applied before first flight |  |  |
| ☐ A.10 | A completed checklist for every child CI in this CI. | Each child CI is cleared on its own. | CM | This checklist set |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ A.11 | Installed configuration record: the serial number and version of every child CI in this CI on the tail. | What is actually installed. | CM | MIL-HDBK-61B Table I |  |  |
| ☐ A.12 | Installation record and installed verification for the CI. | The CI was installed and checked to its drawings. | Quality, Test and Verification | MIL-HDBK-61B Table I; 8.2.1 |  |  |
| ☐ A.13 | Approved RFVs, with use restrictions. A major RFV against this specification is approved by the Government. | Each known departure is approved at the right level. | CM | EIA-649-1A 3.3.2; MIL-HDBK-61B App C |  |  |
| ☐ A.14 | Acquirer approval for every Class I change in effect on the tail. List of open ECPs. | The current approved configuration, approved by the right authority. | CM | EIA-649-1A 3.3.1.5 |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ A.15 | CI index row: designated, Acquirer concurred, ABL established, initial PBL status recorded. | The CI is managed. | CM | Company CI identification process |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist H.  Company-design hardware CI

Example: the control electronics unit. Control tier: Company-spec controlled. The company holds the design.

The company holds full design disclosure for this CI. So this checklist carries the most PBL content.

If the CI hosts loadable software, also complete the software CI checklist for that software.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the CI must be |  |  |  |  |  |  |
| ☐ H.1 | CI performance specification at revision, or its allocated requirements in the parent specification, traced to the parent. | What the CI must do. | Systems Engineering | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ H.2 | Interface requirements, and the ICDs the specification invokes, at revision. | The CI’s connections are defined. | Systems Engineering | EIA-649-1A 3.2.5.1 (2) |  |  |
| ☐ H.3 | Interface register: every row on this CI mapped. No open coverage exceptions. | Every identified interface is controlled. | CM | Company CI identification process |  |  |
| ☐ H.4 | FMEA, FMECA, and hazard analyses for this CI, current to the flight configuration. | The analyses describe the hardware that will fly. | System Safety, RM&SH | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ H.5 | Verification plan, qualification test procedures, and qualification analyses, at revision. | How the CI is shown to meet its specification. | Test and Verification | EIA-649-1A 3.2.3.2.1 (1) b |  |  |
| ☐ H.6 | Design disclosure at revision: drawings, models, and parts lists, with engineering release records. | What the unit was built to. | Design Engineering | EIA-649-1A 3.2.2; 3.2.3.2.2 (1) a; MIL-HDBK-61B 5.4.2.5 |  |  |
| ☐ H.7 | Acceptance definition: the ATP, the critical characteristics, and any first article requirements. | How each unit is accepted. | Test and Verification, Quality | EIA-649-1A 3.2.3.2.2 (1) b, e |  |  |
| ☐ H.8 | Process and material specifications. Definition of special tooling and test equipment. | How the unit was made and tested. | Design Engineering, Manufacturing Engineering | EIA-649-1A 3.2.3.2.2 (1) c, f |  |  |
| Evidence: why the CI is cleared to fly |  |  |  |  |  |  |
| ☐ H.9 | Qualification test reports, tied to the exact configuration tested. | The evidence applies to this design. | Test and Verification | MIL-HDBK-61B App C |  |  |
| ☐ H.10 | ACN at PIN and revision, with current exceptions and limitations. | What the CI may and may not do in flight. | Test and Verification | Company usage; EIA-649-1A 3.3.1.2 (2) w |  |  |
| ☐ H.11 | Qualification-to-flight difference assessment between the qualified unit and this unit. | This unit is covered by the ACN. | Test and Verification, Design Engineering | EIA-649-1A 3.5.3.1 (1) b, applied before first flight |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ H.12 | As-built BOM for this serial number. | What this unit actually is. | Manufacturing Engineering, Quality | MIL-HDBK-61B Table I |  |  |
| ☐ H.13 | ATP results for this serial number. First article inspection record where required. | This unit passed acceptance. | Quality | Record checked against the acceptance definition |  |  |
| ☐ H.14 | Physical verification of the as-built configuration against the as-designed configuration. | The record matches the hardware. | Quality, CM | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ H.15 | Approved RFVs on this unit, with use restrictions. Company authority applies to company-baselined documents. A departure with Class I effect goes through the parent CI. | Each known departure is approved at the right level. | CM | EIA-649-1A 3.3.2; MIL-HDBK-61B App C |  |  |
| ☐ H.16 | Change incorporation record for this unit. List of open ECPs. | The current approved configuration of this unit. | CM | Principle CI-12; EIA-649-1A 3.4.1 (3) |  |  |
| ☐ H.17 | Installation record: tail, position, date, and removal and reinstallation history. | Where the unit is, and where it has been. | CM | MIL-HDBK-61B Table I |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ H.18 | CI index row: designated, Acquirer concurred, ABL established, initial PBL established. | The CI is managed. | CM | Company CI identification process |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist W.  Software CI (CSCI)

Example: the flight control software. Control tier: Company-spec controlled.

For software, the baseline includes the actual product (EIA-649-1A 2.3). The build that flies is the build that must be cleared.

The unit record for software is the load record. It ties a software version to a host hardware serial number.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the CSCI must be |  |  |  |  |  |  |
| ☐ W.1 | Software requirements specification (SRS) at revision, traced to the parent. | What the software must do. | Systems Engineering | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ W.2 | Interface requirements specification (IRS), and the ICDs it invokes, at revision. | The software’s interfaces are defined. | Systems Engineering | EIA-649-1A 3.2.5.1 (2) |  |  |
| ☐ W.3 | Interface register: every row on this CSCI mapped. No open coverage exceptions. | Every identified interface is controlled. | CM | Company CI identification process |  |  |
| ☐ W.4 | Software hazard analysis and software safety requirements, current to the flight build. | The analyses describe the software that will fly. | System Safety | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ W.5 | Software test plan and qualification test procedures at revision. | How the software is shown to meet its requirements. | Test and Verification | EIA-649-1A 3.2.3.2.1 (1) b |  |  |
| ☐ W.6 | Software product specification (SPS), source code, and software version description (SVD) for the flight build. | What the flight build is. | Design Engineering, CM | EIA-649-1A 2.3; 3.2.3.2.2 (1) a |  |  |
| ☐ W.7 | Build environment identification: compilers, tools, and their versions. | The build can be reproduced. | Design Engineering | EIA-649-1A 3.2.3.2.2 (1) g |  |  |
| Evidence: why the CSCI is cleared to fly |  |  |  |  |  |  |
| ☐ W.8 | Qualification test results tied to the exact build, with requirement-to-test traceability and known deficiencies listed. | Every requirement has a passed test step on this build. | Test and Verification | EIA-649-1A 3.5.2.1 (1) c |  |  |
| ☐ W.9 | Configuration of the simulation and lab software used for verification, correlated to its results. | The test environment is known. | Test and Verification | MIL-HDBK-61B App C |  |  |
| ☐ W.10 | ACN at the software version, with current exceptions and limitations. | What the software may and may not do in flight. | Test and Verification | Company usage; EIA-649-1A 3.3.1.2 (2) w |  |  |
| ☐ W.11 | Build-to-flight difference assessment between the qualified build and the flight build. | The flight build is covered by the ACN. | Test and Verification, Design Engineering | EIA-649-1A 3.5.3.1 (1) b, applied before first flight |  |  |
| ☐ W.12 | Listing of developmental software versions in the software library. | Every version that existed is known. | CM | EIA-649-1A 3.5.2.1 (2) i |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ W.13 | Load record: the software version on each host hardware serial number on the tail. | Which build is on which unit. | CM, Test and Verification | MIL-HDBK-61B Table I |  |  |
| ☐ W.14 | Load verification: the installed version confirmed against the SVD. | The record matches what is loaded. | Quality, Test and Verification | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ W.15 | Approved RFVs and open software problem reports that affect flight, each dispositioned. | Each known departure is acceptable for flight. | CM | EIA-649-1A 3.3.2; 3.5.2.1 (1) c |  |  |
| ☐ W.16 | Change incorporation record for the build. List of open ECPs and software change requests. | The current approved configuration of the software. | CM | Principle CI-12; EIA-649-1A 3.4.1 (3) |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ W.17 | CI index row and software identifier register entry for the flight version. | The CSCI and its version are managed. | CM | Company CI identification process |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist P.  Supplier-design hardware CI on a source control drawing

Example: the elevator actuator. Control tier: Company-spec controlled. The supplier holds the detailed design.

The company does not decompose a supplier-designed item. The SCD is the company-level product definition.

Parts inside the item still carry their own records. Life limits, serial numbers, and lot numbers attach to those parts.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the CI must be |  |  |  |  |  |  |
| ☐ P.1 | CI performance specification at revision, or its allocated requirements in the parent specification. | What the CI must do. | Systems Engineering | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ P.2 | Interface requirements and ICDs at revision. The interface definition on the SCD. | The company fixes the interface. The supplier consumes it. | Systems Engineering | EIA-649-1A 3.2.5.1 (2) |  |  |
| ☐ P.3 | Interface register: every row on this CI mapped. No open coverage exceptions. | Every identified interface is controlled. | CM | Company CI identification process |  |  |
| ☐ P.4 | FMEA, FMECA, and hazard analyses current to the flight configuration. Failure modes inside the supplier item are traced to this CI. | The analyses describe the hardware that will fly. | System Safety, RM&SH | EIA-649-1A 3.2.3.2.1 (1) a |  |  |
| ☐ P.5 | SCD at revision: engineering description, qualification requirements, acceptance criteria, critical characteristics, and approved sources. | What the company buys and accepts. | Design Engineering, Supplier Requirements Engineering | EIA-649-1A 2.3 (SCD); 3.2.3.2.2 (1) e |  |  |
| ☐ P.6 | Supplier statement of work. It names the authorship case for serialization, lot recording, and life limits. | Who sets each unit record requirement. | Supplier Requirements Engineering | Company CI identification process |  |  |
| Evidence: why the CI is cleared to fly |  |  |  |  |  |  |
| ☐ P.7 | Supplier qualification report against the SCD. CM logs it as received. The reviewing function accepts it. | The qualification evidence is in hand and accepted. | CM, Supplier Requirements Engineering | Company CI identification process |  |  |
| ☐ P.8 | ACN at PIN and revision. It is valid only for units from a qualified source. | What the CI may and may not do in flight. | Test and Verification | Company usage; EIA-649-1A 3.3.1.2 (2) w |  |  |
| ☐ P.9 | Qualification-to-flight difference assessment, including every supplier change since qualification. | This unit is covered by the ACN. | Test and Verification, Supplier Requirements Engineering | EIA-649-1A 3.5.3.1 (1) b, applied before first flight |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ P.10 | Unit came from an approved source named on the SCD. | The unit is from a qualified source. | Quality, Supplier Requirements Engineering | EIA-649-1A 3.3.1.2 (2) g |  |  |
| ☐ P.11 | Supplier certificate of conformance and acceptance data for this serial number. | The supplier accepted this unit to the SCD. | Quality | Record checked against the SCD |  |  |
| ☐ P.12 | Receiving inspection of the critical characteristics. | The company confirmed what matters most. | Quality | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ P.13 | Unit record: the CI serial number. Serial number and heat lot on each life-limited part inside. Lot number and date code on each lot-recorded part. | What this unit actually is. | Quality, CM | Company CI identification process; MIL-HDBK-61B 5.6.2.1.4 |  |  |
| ☐ P.14 | Life status at installation: cycles or hours consumed against each limit, and shelf life on lot-recorded parts. | No life-limited part flies past its limit. | RM&SH, CM | MIL-HDBK-61B Table I |  |  |
| ☐ P.15 | Traceability chain from each recorded lot to the unit serial number to the tail. | Any part can be found on any aircraft. | CM | Company CI identification process |  |  |
| ☐ P.16 | Approved RFVs on this unit, including supplier variances the company accepted. | Each known departure is acceptable for flight. | CM | EIA-649-1A 3.3.2 |  |  |
| ☐ P.17 | Change incorporation record. List of open ECPs. Supplier change notices dispositioned. | The current approved configuration of this unit. | CM | Principle CI-12; EIA-649-1A 3.4.1 (3) |  |  |
| ☐ P.18 | Installation record: tail, position, date, and removal and reinstallation history. | Where the unit is, and where it has been. | CM | MIL-HDBK-61B Table I |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ P.19 | CI index row with the SCD and supplier statement of work references, and the recording flags and life clock. | The CI is managed. | CM | Company CI identification process |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist G.  Government furnished equipment CI

Example: a government-furnished radio installed in the product structure.

The Government holds the definition of a GFE CI. The company cannot clear it. The company obtains the Government’s clearance and controls the installation and the interfaces.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition: what the CI must be |  |  |  |  |  |  |
| ☐ G.1 | Government identification of the item: PIN, nomenclature, and configuration. | What the Government furnished. | CM | Company CI identification process |  |  |
| ☐ G.2 | Company-side interface requirements and ICDs at revision. | The connections to the GFE are defined. | Systems Engineering | EIA-649-1A 3.2.5.1 (2) |  |  |
| ☐ G.3 | Interface register: the GFE CI appears on a mapped row. No open coverage exceptions. | Every identified interface is controlled. | CM | Company CI identification process (structural floor) |  |  |
| ☐ G.4 | System hazard analysis covers the GFE item and its interfaces. | GFE hazards are controlled at system level. | System Safety | EIA-649-1A 3.2.3.2 (1) a |  |  |
| ☐ G.5 | Installation drawings at revision, with engineering release records. | How the GFE is installed. | Design Engineering | EIA-649-1A 3.2.2 |  |  |
| Evidence: why the CI is cleared to fly |  |  |  |  |  |  |
| ☐ G.6 | Government clearance and limitations for the furnished configuration, obtained and on file. | The Government cleared what it furnished. | Program Management | Contract |  |  |
| ☐ G.7 | Government-furnished data received and logged. | The company has what it needs to install and test. | CM | Contract |  |  |
| ☐ G.8 | Integration test results for the installed GFE, tied to the configuration tested. | The GFE works on this aircraft. | Test and Verification | MIL-HDBK-61B App C |  |  |
| The unit: what is on this tail |  |  |  |  |  |  |
| ☐ G.9 | Installed serial number and the configuration as received. | What is actually installed. | CM | MIL-HDBK-61B Table I |  |  |
| ☐ G.10 | Receipt inspection and installed verification. | The unit matches the Government identification. | Quality | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ G.11 | Discrepancies found at receipt, reported to the Government and dispositioned. | Known problems are resolved by the owner. | CM | Contract |  |  |
| ☐ G.12 | Installation record: tail, position, date, and removal and reinstallation history. | Where the unit is, and where it has been. | CM | MIL-HDBK-61B Table I |  |  |
| Status: is the CM record clean |  |  |  |  |  |  |
| ☐ G.13 | CI index row, GFE control tier. | The GFE CI is recorded. | CM | Company CI identification process |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |

# Checklist N.  Items that fly but are not CIs

Example: flight test instrumentation installed for the test program.

Instrumentation taps buses, adds wiring and power load, and changes weight. It flies, so it needs a decision and a record.

Screen it for CI designation first. If it is a CI, use the checklist for its type. If it is not, use this checklist.

## CI identification

| CI name |  | CI identifier |  |
| --- | --- | --- | --- |
| PIN and revision, or software version |  | Serial, lot, or batch number |  |
| Control tier |  | ACN number and date |  |
| Tail number |  | Installed position |  |
| Flight event or test plan |  | Checklist date |  |

## Checklist

| Done | Artifact | What it proves | Owner | Basis | Document, revision, location | Initials, date |
| --- | --- | --- | --- | --- | --- | --- |
| Definition |  |  |  |  |  |  |
| ☐ N.1 | CI screening result recorded, with rationale. | The designation decision was made, not skipped. | CM | Company CI identification process |  |  |
| ☐ N.2 | Installation drawings, controlled in the parent CI’s documentation, at revision. | The installation is defined. | Design Engineering | EIA-649-1A 3.2.2 |  |  |
| ☐ N.3 | Interface rows for every CI the item connects to: data buses, power, and structure. | Every connection is controlled. | Systems Engineering, CM | EIA-649-1A 3.2.5 |  |  |
| ☐ N.4 | Hazard analysis covers the item: bus loading, power, electromagnetic effects, and structural attachment. | The item adds no uncontrolled hazard. | System Safety | EIA-649-1A 3.2.3.2 (1) a |  |  |
| The unit and status |  |  |  |  |  |  |
| ☐ N.5 | Entry on the tail configuration list, with part number and serial number. | The item is part of the known configuration. | CM | MIL-HDBK-61B Table I |  |  |
| ☐ N.6 | Installed verification against the installation drawings. | The record matches the hardware. | Quality | MIL-HDBK-61B 8.2.1 |  |  |
| ☐ N.7 | Removal plan, and verification that the aircraft returns to its approved configuration after removal. | The aircraft does not keep test configuration by accident. | CM, Quality | MIL-HDBK-61B 8.2.1.1 |  |  |

## Open items and N/A reasons

| Item | Open item or N/A reason | Disposition (closed, on limitations list, N/A) | Owner |
| --- | --- | --- | --- |

## Sign-off

| Role | Function | Name and signature | Date |
| --- | --- | --- | --- |
| Prepared | Configuration Management |  |  |
| Technical review | Systems Engineering |  |  |
| Safety review | System Safety |  |  |
| Test review | Test and Verification |  |  |
| Approved | Program Management |  |  |