CONFIGURATION MANAGEMENT DECISION GUIDE

Sub-Supplier CM Requirements Flowdown by Work Type

Companion to [XXX-CM-PROC-003] §5.8 and the Sub-Supplier CM Flowdown Matrix workbook · In alignment with EIA-649-1A and GEIA-HB-649A

Document No.: [XXX-CM-GUIDE-002]     Revision: [Initial]     Date: [Date]

# 1. Purpose

This guide instructs CM on determining which CM requirements and supplier data requirements (SDRLs) flow down to a sub-supplier, based on the type of work performed. Flowdown that ignores work type fails in one of two directions: over-imposition (a full CM program on a build-to-print machine shop) or under-imposition (no change control on a design agent). The controlling variables are who holds design authority and who performs the build, with item type (hardware, software, COTS, service) as a modifier.

The work-type classification recorded per SSOW is the documented tailoring rationale required by Floor ID F40 (require sub-suppliers to comply with or suitably tailor the same CM requirements imposed on the Supplier) and [XXX-CM-PROC-003] §5.8. The companion workbook (Sub-Supplier CM Flowdown Matrix) holds the operative matrix, SDRL definitions, and the SSOW classification log; this guide explains the reasoning the workbook encodes.

# 2. Work Type Taxonomy

Classify every procurement into one or more of the following. Mixed engagements combine types (e.g., WT2+WT6 for a supplier developing a source-controlled box with embedded software); apply the most demanding matrix cell per requirement row.

| ID | Work Type | Definition | Design Authority / Build |
| --- | --- | --- | --- |
| WT1 | Build-to-Print | Company designs; supplier manufactures to the company-released TDP. | Company / Supplier |
| WT2 | Source-Controlled Design + Build | Supplier develops and builds under a company source control drawing; company controls F3I and qualification; supplier is design activity for item internals. | Split / Supplier |
| WT3 | Design Agent (TDP deliverable) | Supplier develops the design to company SCD/spec; company builds. The deliverable is the TDP itself. | Supplier develops / Company |
| WT4 | Full CI Development-to-Spec | Supplier develops a designated CI to company performance/detail spec and holds design authority at their level; heaviest flowdown — comply-or-tailor EIA-649-1A. | Supplier / Supplier |
| WT5 | COTS / Catalog Item | Vendor holds design; company controls via vendor item control drawing; flowdown is minimal and mostly notification. | Vendor / Vendor |
| WT6 | Software / Firmware Development | CSCI or embedded-code development; adds software CM plan, developmental configuration, library control, version identification, SVD/VDD, build reproducibility. Usually combined with WT2 or WT4. | Supplier / Supplier |
| WT7 | Repair / Overhaul (MRO) | Supplier maintains fielded configuration: as-maintained records, retrofit incorporation (F33), unit-identifier and life retention through PIN changes. | Company / Supplier (repair) |
| WT8 | Special Process Services | Process-only services (heat treat, coating, NDI); no item design; flowdown is process-spec revision control and process-change notification (frozen planning). | Company (process spec) / N/A |

# 3. Flowdown Principles

## 3.1 Requirements Follow the Work, Not the Supplier

The matrix keys each CM requirement to work type: change and variance control scale with design authority; identification, traceability, reconciliation, and FAI scale with who builds; release-system discipline scales with who produces documentation. A supplier performing two work types receives the union of both columns.

## 3.2 Audit Obligations Follow Designation, Not Work Type

FCA/PCA is conducted for designated CIs per [XXX-CM-PROC-002] §5.2.1 and Appendix C — designation is contractual or risk-based, never automatic. Work type therefore determines the supplier’s role if the item is or becomes designated, not whether audit obligations exist:

- WT2/WT4 conduct the CI-level FCA/PCA at their facility — ceremony may be tailored (internal co-chair/certification), substance may not: requirements-to-test traceability matrix (F44), non-compliance documentation (F43, F31), PPSL and installed-parts verification (F30), and FCA/PCA-unit difference identification (F34) are performed in full.

- WT6 conducts the CSCI FCA and the witnessed build/installation verification against the SVD at PCA.

- WT1 supports the PCA on their build: facility access, as-built records, PPSL evidence, manufacturing instructions.

- WT3 provides data support only (TDP, traceability matrix inputs). WT5/WT7/WT8: generally not applicable.

Designation can arrive or change after award: the Appendix C assessment re-runs on triggers, and awarding to a new sub-supplier is itself trigger C3/C4 — placing the subcontract can cause the designation that imposes audit obligations on it. Every SSOW where designation is possible therefore carries a standing conditional audit clause (“if the item is or becomes designated per [XXX-CM-PROC-002] §5.2.1, supplier shall perform/support the audit per its work-type role, deliverables per SDRL S-10 through S-12”) with the associated SDRLs cited as triggered deliverables. Deciding audit applicability once at RFP time converts every post-award designation into a contract-modification negotiation from a position of weakness.

## 3.3 The At-Scale Machinery Applies Regardless of Designation

Configuration verification is continuous and embedded; audits are discrete validations of it. “No audit designation” never means “no verification flowdown”: the reconciliation, CSA, traceability, and FAI rows apply by work type to every supplier that builds. Two reconciliation flowdowns are distinct and both required for builders:

- Reconciliation process documentation (SDRL S-02): capability evidence — how the supplier reconciles as-built to released design at scale. Reviewed with the CM plan; input to the §5.8 effectiveness determination and to Appendix C item C7 when assessing supplier-built CIs.

- Reconciliation report (SDRL S-08): the recurring per-delivery/per-lot output — as-built vs. released design revision, open deltas with disposition, incorporated changes with effectivity — expressed in Part Traceability Record identifiers. For WT7, the as-maintained equivalent including retrofit incorporation records (F33).

## 3.4 Part Traceability Record

Defined once (SDRL S-06) and referenced wherever identifiers matter: serial numbers; lot/batch numbers; life-limited part designation with assigned life limits and life consumed at delivery; safety-critical designation; special handling designations; all traceable to next higher assembly (F39). Unit identifiers and consumed life survive PIN changes and overhaul. Authorship differs by work type: WT1 receives designations in the TDP and marks, records, and reports against them; WT2/WT4/WT6 propose designations during design development for company approval — state this in the SSOW or a WT2 supplier will wait for the company to provide what the company is waiting for the supplier to propose. WT5 passes through vendor life/safety data where the catalog item carries them.

## 3.5 Floor Items Do Not Dilute at Lower Tiers

Matrix rows tagged with Floor IDs are CM floor requirements. On safety-significant product lines, a proposal to tailor or omit a floor row for a supplier in scope is dispositioned through the CM floor gate ([XXX-CM-PROC-003] §5.1.1) and recorded in the program CM Floor Compliance Matrix — a floor item may not be silently dropped at a lower tier (F40). Recursive flowdown applies wherever the supplier has sub-tiers of its own.

# 4. Procedure

- Classify: at SSOW development, CM classifies the procurement by work type(s) and records the classification on the SSOW Classification sheet of the companion workbook. The classification is the F40 tailoring rationale.

- Derive: read the matrix column(s) for the selected work type(s); for mixed types apply the most demanding cell per row. Resolve every C (conditional) cell against its stated condition, including the audit rows against current designation status.

- Cite: incorporate the required/tailored requirements and their SDRLs into the SSOW; include the standing conditional audit clause wherever designation is possible; document any tailoring of floor rows via the §5.1.1 gate.

- Sign: CM reviews and signs the SSOW prior to release (the CM SSOW review metric counts this signature; define whether it attests review-occurred or content-adequacy per the metric definition).

- Maintain: re-run the classification on scope change, work-type change, or audit-designation change (including designations triggered by the award itself); update the SSOW and the classification log.

# 5. References

- EIA-649-1A §3.(2), §3.(4) — supply chain CM flowdown and audit support (F40, F41)

- [XXX-CM-PROC-003] Configuration Management Planning and Management, §5.8

- [XXX-CM-PROC-002] Configuration Audit Process, §5.2.1 and Appendix C (audit designation)

- [XXX-CM-PROC-001] CI Identification Process, §5.5 (interfaces), §6 (records)

- Sub-Supplier CM Flowdown Matrix workbook (operative matrix, SDRL definitions, SSOW classification log)

- CM Floor Register and CM Floor Operating Model; SM-0001 Safety Management System Standard

This guide is a decision aid; contract requirements govern where they differ. Final audit designation authority for Acquirer-controlled CIs rests with the Acquirer.