  - CONFIGURATION MANAGEMENT REQUIREMENTS

Principle CM-1: Configuration Management implementation requires a balanced and continuous application of CM functions and their underlying principles throughout the product life cycle.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

The Supplier shall comply with CM requirements in this document as tailored in the contract.

NOTE: This requirement must be included as part of the contract.

- The Supplier shall require its Sub-suppliers to comply with or suitably tailor the same CM requirements that are being imposed upon the Supplier.

- The Supplier shall establish and execute a CM process to control all configuration documentation representing or comprising the configuration item.

- The Supplier shall support the Acquirer’s requests to audit and verify Supplier and Sub-supplier CM process requirements.

    - Planning and Management

Principle CMP-1: The foundation for CM planning, which delineates the specific CM application methods and their levels of emphasis, is an understanding of the context and environment of the product to which the CM process is to be applied.

Principle CMP-2: CM Planning documents how the organization will implement CM throughout the applicable phases of the product life cycle to provide consistency between product configuration information and the product.

Principle CMP-3: To implement planned CM functions, resources are identified and applied and responsibilities to perform CM activities are assigned.

Principle CMP-5: CM Procedures document how each CM function is implemented to accomplish the intent of the CM planning.

Principle CMP-6: Provide CM training to assure that individuals understand their responsibility, authority, accountability, and the procedures for performing CM.

Principle CMP-7: Periodic assessment of the effectiveness of CM procedures and tools and of compliance with the Configuration Management plan maintains the health of the CM process.

- Supplier shall perform CM planning to define the following factors:

  - Objectives of the CM program and of each applicable CM element.

  - Appropriate level of CM activity for each CM function throughout the product’s life cycle.

  - CM organization and organizational relationships.

  - Responsibilities and authority of CM professionals.

  - CM resources to be used (tools, techniques, training, and methodologies).

  - Coordination activities to be used with internal and external agencies (e.g., Acquirer, other Suppliers, other Government agencies, foreign governments).

  - Functions, responsibility, and authority of CCBs.

  - A release process for product configuration documentation.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Methods used to determine the effectiveness of Sub-supplier/Sub-vendor CM processes, to include actions when determination of CM processes are ineffective.

  - CSA activities for creating, editing, reviewing, approving, releasing, publishing, and distributing product configuration documentation and configuration change documentation.

  - Identification and labeling structure for Configuration Items.

NOTE: The Acquiring Activity may cite DI-SESS-80858 Supplier's Configuration Management Plan and DI-MISC- 80508 Technical Report–Study/Services, for specifying the delivery of the data product that emanates from meeting this requirement.

      - Monitoring Configuration Management Performance

Principle CMP-4: The establishment and maintenance of CM performance and status measurements are necessary for all products where compliance with the Configuration Management planning and continuous improvement are critical.

The purpose of monitoring an organization’s CM performance is to assess, appraise, and evaluate the value of its CM processes in order to provide continuous improvements of the processes. Assessments can be conducted with Acquirer oversight and agreed upon by both parties during the project’s life cycle.

- The Supplier shall include in their Configuration Management Plan (CMP) a methodology for generating, measuring, and reporting CM performance data.

- The Supplier shall make CM performance data available to the Acquirer.

Suggested performance metrics may include but are not limited to:

  - Number of open changes and variances awaiting Acquirer disposition

  - Number of closed Supplier changes and variances

  - Number of minor changes/variances

  - Number of major changes/variances

  - Change processing cycle time

  - Average amount of time (days) changes or variances remain open

  - Percentage of requests for change/variance by category, nomenclature, Configuration Item identifier number, etc.

  - Change request rejection rate and reason for rejection

      - Supplier’s Configuration Management Plan

Principle CMP-8: Performing configuration management includes responsibility for the configuration management performance of the supply chain.

(1) Supplier shall submit to the Acquirer for review and approval the CM Plan describing the processes, methods and procedures used to manage the functional and physical characteristics of the assigned CI(s) for the life of the contract and shall include the CM performance of Sub-tier suppliers.

NOTE: The Acquiring Activity may cite DI-SESS-80858 Supplier's Configuration Management Plan, for specifying the delivery of the data product that emanates from meeting this requirement.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Technical Reviews

- The Supplier shall provide configuration documentation to support each Technical Review.

These reviews may include the following:

  - Alternative Systems Review (ASR)

  - System Requirements Review (SRR)

  - System Functional Review (SFR) (establishing the Functional Baseline)

  - Preliminary Design Review (PDR) (establishing Allocated Baseline)

  - Critical Design Review (CDR) (establishing the Product Baseline (initial))

  - Test Readiness Review (TRR)

  - System Verification Review (SVR)

  - Production Readiness Review (PRR)

  - Other technical reviews, whether periodic or ad hoc, as prescribed by the contract

NOTE: Refer to reference IEEE 15288.2 Technical Reviews and Audits on Defense Programs for how the reviews are to be conducted.

      - Product Configuration Documentation Processes

Principle CMP-9: Information processes, including collection and processing, controlling status, providing interoperability and exchange, and long term preservation, are essential elements of effective CM planning and management.

- The Supplier shall address contractually specified product configuration documentation requirements in its CMP, especially data handling, processing, storage, retention, integrity, transfer, releasability/disclosure, security, and maintenance documentation requirements.

- The Supplier shall inform the Acquirer of any requirements that will be imposed on the Acquirer to maintain the security and integrity of shared data.

    - Configuration Identification

Principle CI-1: Configuration identification is the basis from which the configuration of products is defined and verified; products and their product configuration information are labeled; changes are managed; and traceability is maintained throughout the product’s life cycle.

Principle CI-2: Product configuration information serves as the basis for development, production, operation, and maintenance/support of the product.

Principle CI-3: Enterprise identifiers designating the responsible designer, manufacturer, or preparer provide uniqueness to the identifiers of products and product configuration information.

Principle CI-4: Product identifiers are assigned so that one product can be distinguished from other products; one configuration of a product can be distinguished from another and the correct corresponding product information can be retrieved.

Principle CI-5: Individual units of a product are assigned a unique product unit identifier when there is a need to distinguish one unit of the product from another.

Principle CI-6: When a product is modified, it retains its original product unit identifier, even though its part identifying number is altered to reflect a new configuration.

Principle CI-7: A series of like units of a product is assigned a unique product group identifier when it is unnecessary or impractical to identify individual units, but necessary to correlate units to a process, date, event, or test.

Principle CI-8: Product configuration information is uniquely identified so it can be referred to precisely and is made accessible.

Principle CI-9: Establishing product structure to organize product configuration information provides efficient application of the information over its lifecycle.

The purpose of Configuration Identification is to establish and maintain a definitive basis for control and status accounting for a product and associated configuration documentation as it evolves throughout its life cycle. The responsibility for configuration documentation information is passed between design engineering and CM as it is developed, analyzed at design reviews, and then officially managed by CM to become part of formal baselines.

- The Supplier shall use a documented process for configuration identification.

- The Supplier shall accomplish the following for hardware, software, and services:

  - Apply the criteria identified in 3.2.1 to select Configuration Items.

  - Establish a Configuration Item structure and hierarchy.

  - Select configuration documentation to be used to define configuration baselines for each CI.

  - Identify and control interfaces.

  - Assign identifiers (i.e., enterprise, product, unit, group, and document) to Configuration Items and their component parts and associated configuration documentation. This will include DAI and PIN including prefixes and suffixes, and serial, lot, or batch numbers will remain the same, with a specific Acquirer type designation, as appropriate.

  - Whenever an item is modified to a new configuration that is not interchangeable with the existing configuration, the part or identifying number (PIN) must change. Although the PIN is changing, the part must retain its original unit identifier (batch, lot, or serial number) to provide traceability to the original configuration.

  - Include the CAGE Code of the design activity for hardware and software and affix those CAGE codes to all Configuration Items, their subordinate parts and assemblies, configuration documentation, software media, and products.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

  - Place each item of configuration documentation (e.g., item, material, or process specifications) and software specification, Software Version Description (SVD), etc. under configuration control.

  - Establish and manage the functional, allocated and product baselines at the appropriate points in the system/Configuration Item life cycle.

  - Mark or label items and documentation with their applicable identifiers that correlates to the item, configuration documentation and other associated data.

  - Obtain Acquirer approval of the type designation and nomenclature for each Configuration Item that is designated by the Acquirer for control, tracking and logistics purposes.

NOTE: Refer to MIL-STD-196 Joint Electronics Type Designation System, MIL-STD-1464 Army Nomenclature System, MIL-STD-1661 Mark and Mod Nomenclature System, ASME Y 14.100 Engineering Drawing and Related Documentation Practices.

NOTE: Refer to DODI 5230.24 Distribution Statements on Technical Documents.

NOTE: The Acquiring Activity may cite DI-SESS-81254 Request for Nomenclature, DI-SESS-81121 Baseline Description Document, for specifying the delivery of the data product that emanates from meeting this requirement.

- In addition to the list above, the Supplier shall accomplish the following for software:

  - Assign system product-unique identifiers to elements of the software development environment such as Commercial-Off-the-Shelf (COTS) development products like compilers, linkers, loaders, binary developmental libraries, configuration settings, automation scripts, and other elements that are part of the system and will conform to the format specified in the CMP.

  - Provide a part number representing the device with the embedded code (in cases where both the hardware device and the embedded code are controlled via a single engineering Altered Item Drawing).

  - Part number and the software medium is labeled with the Supplier’s code/name identification and marked separately on the device.

  - Designate computer software as Configuration Items.

NOTE: The Acquiring Activity may cite DI-SESS-81011 Drawing/Model Number Assignment Report, for specifying the delivery of the data product that emanates from meeting this requirement.

      - Configuration Item Selection

- The Supplier shall select and submit potential CIs for Acquirer review and selection concurrence.

- The Supplier shall identify as a Configuration Item any item that satisfies a system end use function and designates it for separate CM because the item meets or impacts one or more of the following:

  - An item critical to the functioning of the system.

  - An item whose failure might result in serious injury, loss of life or loss of primary system capability.

  - An item whose technology is complex, not well known, and critical to meeting acquisition, operational or supportability goals of the program.

  - Is under control as designated by the Acquirer.

  - Requires separate type designation, versions, or nomenclatures.

  - Is critical or high risk and failure would have significant financial impact.

  - Requires any one or more of the following logistics elements:

    - Supply support Allowance Parts List (APL)

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Supply support Allowance Equipage List (AEL)

    - Supply support Allowance Component List (ACL)

    - Test equipment (e.g., General Purpose, Special Purpose, or Built-in)

    - Technical manuals

    - Planned Maintenance System (PMS) Maintenance Index Pages (MIP) and Maintenance Requirement Cards (MRC)

    - Danger or Caution Tag Out requirements

NOTE: The Acquiring Activity may cite DI-SESS-81254 Request for Nomenclature and DI-SESS-82007 Configuration Item Documentation Recommendation, for specifying the delivery of the data products that emanates from meeting this requirement.

      - Engineering Release System

Principle CI-10: Product release provides the appropriate process(es) and evidence necessary to validate the integrity of the product for its intended use.

- The Supplier shall use an Engineering Release process, which employs formal release records to authorize the use of new or revised configuration documentation.

- The Supplier shall use an engineering release system to issue initial and subsequent approved, including minor changes to the product configuration documentation.

- The Supplier shall use an engineering release process to authorize the use of configuration documentation associated with an approved configuration.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`

- The Supplier shall incorporate approved configuration documentation and Engineering Release Record into their CSA system.

NOTE: The Acquiring Activity may cite DI-SESS-80463 Engineering Release Record for specifying the delivery of the data product that emanates from meeting this requirement.

      - Configuration Baselines Types

Principle CI-11: A configuration baseline is established by agreeing to the definition of the attributes of a product at a point in time and identifies a known configuration to which changes are addressed.

Principle CI-12: A baseline for any product, or any document, plus the approved changes, is the current approved configuration.

There are three major configuration baseline types established during the life cycle of a product: Functional (FBL), Allocated (ABL) and Product (PBL). The FBL and ABL are part of the design requirements development process and used as inputs to the detailed design realization process. The PBL constitutes the final output of the design realization process. Each of these baselines must be maintained and configuration controlled by the acquiring activity or supplying activity as required by the contract.

        - Configuration Baselines and Their Configuration Documentation

- The Supplier shall generate the configuration documentation required for the configuration baselines being established by the Acquirer, as tailored in the contract.

NOTE: The Acquiring Activity may use MIL-STD-31000 Technical Data Packages and its associated Data Item Descriptions (DIDs) for specifying the delivery of the data product that emanates from meeting this requirement.

- The Supplier shall trace each succeeding baseline to its predecessor(s) including a detailed extension.

        - Functional Configuration Documentation (FCD)

- The supplier shall document the FBL of a system by:

  - Describing its functional, performance, interoperability, and interface requirements.

  - Describing the verifications required to demonstrate achievement of those requirements.

  - Identifying any specialized software and documenting the operating environment used to author the above requirements.

NOTE: The FBL is made up of the contract and the A-level specification, End Item, or System level specification.

          - Allocated Configuration Documentation (ACD)

- The Supplier shall document the ABL of a Configuration Item by:

  - Describing its functional, performance, and interoperability requirements that are allocated from those of a system or higher-level Configuration Item, and interface requirements with interfacing Configuration Items.

  - Describing the verifications required to demonstrate achievement of those requirements.

  - Identifying any specialized software and documenting the operating environment used to author the above requirements.

          - Product Configuration Documentation (PCD)

- The Supplier shall document the PBL of a Configuration Item by:

--`,`,,,```,````,```,``

  - Describing its detailed design including necessary physical (form, fit, and function) characteristics and selected functional/performance requirements selected for production test, qualification, and acceptance test.

  - Identifying the verifications necessary for accepting product deliveries (first article and acceptance instructions).

  - Identifying and documenting the design of any special tooling, software, equipment, and facilities required to manufacture, operate, maintain, calibrate, or inspect items contained in the design.

  - Identifying and documenting the design of any special packaging parts required to package the CI.

  - Including any form, fit, function and quality assurance provisions required to accept deliveries of the Configuration Item (first article or acceptance inspection).

  - Including any unique process specifications required to manufacture, operate, maintain, or calibrate items contained in the design.

  - Identifying any specialized software and documenting the operating environment used to author the detailed design.

  - If required by contract, may include technical data, which provides instructions for the installation, operation, maintenance, training, and support of a system or equipment.

  - Identifying and documenting any COTS hardware or software used in the detailed design.

- The Supplier shall document the PBL of a privately developed Configuration Item by including any verification provisions required to accept deliveries of the Configuration Item (first article or acceptance inspection).

        - Maintenance of Configuration Documentation

(1) The Supplier shall maintain the current approved configuration documentation by creating a new revision to configuration documentation after the change approval, unless it applies to only TDPs controlled by the Supplier, or unless otherwise stated in the contract or agreement. The new revision to the configuration document shall provide traceability by detailing conditions before and after the change is incorporated and by identifying the change as the revision authority.

      - Configuration Identifiers

- The Supplier shall uniquely identify each CI. Each Configuration Item and component shall be assigned a unique product identifier (e.g., PN/PIN).

- The Supplier shall determine the types of configuration documentation required for each Configuration Item and component.

- The Supplier shall assign unique identifier and other identifiers for the Configuration Item and to the technical documentation that comprises the Configuration Item configuration documentation.

- The design document identifier (i.e., CAD model, engineering drawing) shall be the same as, or traceable to, the PN/PIN.

        - Product Identification/Marking

- The Supplier shall identify all Configuration Items including their sets, groups, units, assemblies, subassemblies, parts, or other items by marking in accordance with MIL-STD-130, MIL-STD-1285, or MIL-STD-13231, as applicable.

- The Supplier shall adhere to DoD Item Unique Identification (IUID) directives, instructions, and standards as tailored in the contract.

NOTE: For DoD IUID directives, instructions and standards for each applicable Unique Identification and Unique Item Identifier CI, refer to MIL-STD-130.

- When required in acquisition documents, IUID marking shall be in accordance with MIL-STD-130. Identification marking may be applied direct to an item or by separate means in accordance with MIL-DTL-15024.

- For Configuration Items marked with Unique Item Identifier (UII), the Supplier shall check DoD IUID Registry to determine that the UII has not been previously assigned to an item.

        - Software Marking and Labeling

- The Supplier shall mark and label software as follows:

  - Embed software identifier and version in the source and executable code header including technical and administrative.

  - Mark each software medium (e.g., disks, hard drives) with a label to include distribution statement, ITAR statement and classification and that all identifiers shall be repeated on the media, as applicable.

  - Each label shall contain, or provide cross-reference to, a list of the applicable software identifiers of the entities it contains.

  - The label shall also indicate the status of the software maturity on the medium (i.e., whether this software is a release candidate or if the software has been tested and verified).

  - Mark with a label all Computer Software Configuration Items deliverable media. Label shall contain:

    - Acquirer Contract number.

    - Computer Software Configuration Items Number (include a designator indicating whether source or executable code).

    - Design activity CAGE Code.

    - Media Number (e.g., 1 of 2, 2 of 2) if there are multiple units per set. Each time a new version of software is issued, new copy numbers, starting from one, must be assigned.

      - Interface Management

Principle CI-13: Interfaces between products are managed by mutually agreeing to defined common product attributes, making them part of the product configuration baselines for each product, and applying a process to maintain interface integrity.

Principle: CI-14: Configuration items are designated where distinct control is appropriate.

        - Interface Identification and Documentation

- The Supplier shall identify the internal and external interface requirements for the system and its Configuration Items.

- The Supplier shall incorporate into the FCD, ACD, and PCD all interface information.

- The Supplier shall maintain configuration control of interface requirements defined in baseline specifications.

NOTE: The Acquiring Activity may cite DI-SESS-81248 Interface Control Document, for specifying the delivery of the data product that emanates from meeting this requirement.

        - Interface Control Working Group (ICWG) Membership

- Each interfacing Supplier shall support and provide a representative to the Acquirer’s ICWG.

- The ICWG Supplier representative shall be empowered to:

  - Commit the interfacing Sub-supplier to specific interface actions and agreements.

  - Provide assigned and agreed-to draft interface control documentation at a specified period prior to the Acquirer’s ICWG meeting where it will be discussed.

  - Update, release, control, and re-release interface control documentation reflecting the ICWG decisions.

  - Distribute copies of such released interface control documentation to other ICWG participants.

    - Configuration Change Management

Principle CCM-1: Changes to an approved configuration are accomplished using a systematic, measurable change process.

- The Supplier shall apply configuration control to each Configuration Item and its components and configuration documentation.

- The Supplier shall confirm that:

  - Configuration baselines are maintained and controlled.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Configuration identification control of all Configuration Items and their associated configuration documentation is maintained.

  - Product configuration changes and variances are documented, coordinated, evaluated, dispositioned, and recorded in a configuration status accounting system and reported to the Acquirer.

  - Requested configuration changes and variances address all areas of impact, to include cost, operational, sustainment, implementation actions (e.g., ordering material, designing tooling, manufacturing planning, and interface design, support equipment, software code, producing parts, retrofit).

  - Released/approved configuration changes are incorporated into all product configuration information including the CM/status accounting system, and implemented into each product/CI/component impacted in a timely manner.

  - Released/approved configuration changes are verified as being incorporated in product configuration information to maintain control and consistency, as well as, prepare release authorization documentation.

  - Released/approved variance corrective actions are implemented and verified.

- Supplier shall support a joint Acquirer/Supplier Configuration Control Board(s).

      - Engineering Change Proposal (ECP)

(1) The Supplier shall use an ECP to document changes to the approved baseline(s) via their configuration documentation and submit to the Acquirer or Acquirer’s designated change authority.

        - Engineering Change Process

--`,`,,,```,````,```,``,,,,,,`,

Principle CCM-2: Justifying the need for a change provides the rationale to commit resources required to document, process, and if approved, implement the change.

Principle CCM-3: A unique change identifier enables tracking of the request for change and the status of implementation and verification of the approved change.

- When an engineering change affects documentation previously approved by the Acquirer, the Supplier shall perform the following actions:

  - Define the need for the change.

  - Review product model and other technical data to determine what specific changes will need to be made.

  - Prepare an ECP, Notice of Revision (NOR) and technical data supporting the change.

  - Prepare a separate ECP for each engineering change, which has its own distinct objective.

  - Incorporate the approved ECP into the configuration documentation in a timely manner (unless otherwise stated in the contract, approved ECPs will be incorporated into the configuration documentation upon ECP approval.)

  - Implement and update configuration documentation with approved changes described in the ECP, using a documented engineering release system.

  - Record all changes in the CSA system database.

  - Verify that the approved change was implemented correctly.

  - Identify superseding revisions to ECPs. An ECP must be revised when alterations or changes to the initial ECP are necessary.

  - Provide an implementation timeline schedule for each ECP.

  - State in ECP any effects on the program in case the ECP is not approved.

  - Notify the Acquirer when there are changes to Commercial off the Shelf (COTS) hardware or software via an ECP that may affect interface or interoperability characteristics.

        - Classification and Authority of ECP

Principle CCM-4: Classification of a requested change determines the appropriate level of review and the applicable change approval authority.

- The Supplier shall use content in “DD Form 1692” or equivalent for assigning a Major (Class I) or Minor (Class II) classification to an ECP.

- An ECP shall be classified as a Major (Class I) change, after the establishment of the product baseline, when it has one or more combination of the following criteria:

  - A change to approved configuration documentation for which the Acquirer is the Current Document Change Authority or that has been included in the contract or statement of work by the tasking activity.

  - A change, which affects any physical or functional requirement in approved functional or allocated configuration documentation.

  - A change which affects or changes any approved acquisition baseline (FBL, ABL, and PBL) that is owned by the Acquirer.

  - A change which affects compatibility with interfacing products (including such products as test equipment, support equipment, software, and products furnished by an Acquirer) or that affects one or more of the following:

    - Delivered operation or servicing instructions of products, assemblies, or components.

    - Required calibration to the extent that product identification should be changed.

  - A change, which affects or changes the current system requirements or any specification type. This may include but not be limited to the following:

    - A change or correction to an approved program or project spe-c-`,if`,i,c,`a``t,i`o``n`,``o`,r``,e,,q,,u,`,i`v-`a`,l,e`,n,,,t-;``,,`,,,,---

    - The software requirements specification;

    - Any interface characteristics and documents for those requirements or specifications;

    - Any imposed qualification and acceptance requirements;

    - Any other imposed technical requirements definition documents;

    - A change which affects or changes specified and approved requirements including:

      - Safety;

      - Reliability;

      - Maintainability;

      - Durability;

      - Survivability;

      - Supportability; and

      - Quantitative requirements that result in product attributes that would be outside specified limits or specified tolerances.

  - A change which affects required interchangeability (form, fit, function (F3I)), substitutability and replaceability of products, assemblies, or components at the end item or non-repairable subassembly level;

  - A change which affects sources defined by a Source Control Drawing;

  - A change which affects any software build, version or release of the operational, test and checkout or maintenance software;

  - A change which affects software or hardware interface compatibility;

  - A change which affects operational, test, or maintenance computer programs;

  - A change which affects the required hardware’s form, fit, or function, its interface, or the software version of the embedded code;

  - A change which affects operator and maintenance manuals (when the change is material);

  - A change which affects or changes the hardware or software of maintenance and operating training systems;

  - A change which affects Security such as: Informational (INFOSEC), Operations (OPSEC) and or Cyber (CYBER SECURITY);

  - A change which has significant environmental Impact;

  - A change which affects or changes the contractual Life cycle costs and savings associated with the current system or an existing ECP;

  - A change which affects or changes user skills or user physical attributes;

  - A change which affects contractual work performance and schedules;

  - A change which requires retrofit of delivered products (e.g., by pro--`d,`u,,c,`t``,r`e``c`,`a``l,l`,`,m,,,,o,`d,`i-f`i`c,,a`,,t,i,o-`n`,,`k,,i,t,-i-n- stallation, attrition, replacement during maintenance using modified spares);

  - A change which affects or changes weight, balance, moment of inertia (when a factor);

  - A change which affects or changes electromagnetic characteristics (when a factor);

  - A change which affects, changes, or has a significant impact to logistical support requirements such as training, technical or operational manuals, spares, maintenance procedures, or equipment, etc.;

  - A change which requires or mandates that a re-qualification of the item undergoing the change must occur;

  - A change which requires a new or updated Domain certification (e.g., airworthiness, sea worthiness, etc.) to occur;

  - A change which affects or changes Biomedical factors;

  - A change which affects or changes personnel manning;

  - A change which corrects deficiencies;

ab. A change, which affects or changes interoperability requirements;

ac. A change, which changes the operational capabilities or logistics supportability of the system or item, and the change is significant and measurably changes the system effectiveness;

ad. A change which prevents slippage in an approved production schedule, (if contractually affected);

ae. A change to approved configuration documentation that incorporates or integrates COTS hardware or software;

af. A change which changes or impacts non-technical issues such as fees, incentives, costs, cost schedules, price, and delivery to customer(s), including incentives and fees, guarantees, warranties, contracted deliveries, schedule milestones, or contract milestones.

    - Minor (Class II) ECPs do not impact the criteria above for Major (Class I) changes.

    - For military derivatives of commercial products regulated by the Federal Aviation Administration (FAA) under Code of Federal Regulations (CFA), Title 14, Chapter I, Subchapter C, Part 21, Subpart D – Changes to Type Certificates; Major/minor classification may have a different meaning and criteria with regard to FAA certification.

        - ECP Format

Principle CCM-5: As the primary vehicle for referencing and managing a change, the request for change must be clear and comprehensive from technical, cost, and scheduling perspectives.

- The Supplier shall use appropriate form/text, such as "DD Form 1692,” or equivalent for referencing and managing a change. Unless otherwise negotiated between Acquirer and Supplier, the Supplier shall prepare ECPs on DD Form 1692-series of forms or its own forms provided that the information requirements of DD Form 1692-series are satisfied.

- The Supplier shall obtain approval from the Acquirer if the Supplier wants to use its own ECP format for Major (Class I) and Minor (Class II) ECPs.

NOTE: The Acquiring Activity may cite DI-SESS-80639 Engineering Change Proposal, for specifying the delivery of the data product that emanates from meeting this requirement.

        - Supporting Data

Principle CCM-6: Prior to approval, a requested change is evaluated for all impacts and risk considerations including technical, operational, support, schedule, and cost, as well as the consequ--e`,n`,,c,`e``s,``o``,f``n`,o``,t,,a,,p,`,p`-r`o`,,v`,i,n,,g-``t,,h`,e,,,r--e- quest.

- The Supplier shall support ECPs with drawings and other data (e.g., Logistics Product Data (LPD), detailed cost proposal data, test data and analyses, quality, packaging, interchangeability factors) to better understand and evaluate the change and its impacts upon the system, its operation and support.

- The Supplier shall include in the ECP implementation production and retrofit costs, when the contract references life cycle cost and/or operation and support cost model, the following costs:

  - All future production and spare items projected to be procured for the program.

  - All projected operation and support costs for operation of the total inventory of items by the Acquirer.

- The Supplier shall include in the ECP, as supporting data, a summary of any testing done by the Supplier to validate concepts or new technology to be employed in the proposed change and details of such test data vital to the decision regarding acceptance of the change.

        - ECP Decisions

Principle CCM-7: After considering all impacts and risk factors, change approval decisions are made by an appropriate authority who can commit resources to implement the change.

NOTE: Usually proposed ECPs (regardless of class) are not implemented until an approval has been obtained by the designated Configuration Approval Authority. If the Supplier chooses to proceed with implementation prior to that approval then they do so at their own risk with the possibility that any such implementation work will have to be undone or scrapped, at their own cost, if the proposed change is disapproved.

Principle CCM-8: An approved change is implemented in accordance with documented direction approved by the appropriate level of authority.

The Acquirer is configuration approval authority for all Major (Class I) engineering changes for the Configuration Item once the Acquirer has configuration control of the baseline.

        - Engineering Change Priorities

The Acquirer uses the engineering change priority recommendation of emergency (48 hours), urgent (30 days), or routine (90 calendar days) to determine the time frame in which the ECP is to be reviewed, evaluated, ordered, and implemented.

(1) The Supplier shall assign an Emergency or Urgent Engineering Change Priority to a Major (Class I) ECP with prior Acquirer approval.

        - Related Engineering Changes

A related engineering change occurs when the desired engineering change in on-e-`,i`t,e,,`m``,`(``t`h,`e``,`b`,a,,,s,,i`c,`-o``r,,`o,,r,,i-g``i,n,`a,,,l,-e--ngineering change) requires related engineering changes in other items to retain (or attain) either an interface match or compatibility and interoperability of the associated items.

          - Related Engineering Changes - Single Prime, Single Procuring Activity

(1) The single prime/procuring Supplier shall include all support data and interface items in one basic/original uniquely numbered engineering change.

          - Related Engineering Changes - Single Prime Multiple Procuring Activities

- The Supplier shall assign a number to the basic ECP.

- The Supplier shall use the basic ECP number, plus a separate dash number, for related ECPs submitted to other procuring activities.

          - Related Engineering Changes - Separate Primes

- The Supplier shall submit a coordinated basic ECP when a desired engineering change in one product (the basic ECP) requires related engineering changes in other products that are the responsibility of other prime Suppliers who are participating in a specific item development or production program.

- The Supplier’s coordinated basic ECP shall include data showing the extent of the coordination and its results to the related ECPs of the other prime Suppliers.

- The Supplier shall include in the basic ECP the impact on the other items.

- The Supplier shall cross-reference basic and other related ECPs.

        - Minor (Class II) ECP

(1) The Supplier shall assign an ECP classification of Minor (Class II) to an ECP whose engineering change does not meet the criteria for a Major (Class I) change. Minor (Class II) ECPs are limited to administrative, documentation clarifications, corrections, or configuration changes, which do not affect interchangeability.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Concurrence in Classification

- The Supplier shall obtain Acquirer classification concurrence of any Minor (Class II) ECP. The Acquirer may delegate authority to an Acquirer’s designated representative, Supplier or retain within the government, for the concurrence of classification for Minor (Class II) ECPs as tailored in the contract.

- The Supplier shall cancel a Minor (Class II) ECP if the Acquirer disagrees with the classification of the ECP. The Supplier may revise and resubmit the proposed change as a Major (Class I) ECP.

- The Supplier shall submit to the Acquirer for final decision, any classification disagreements between the Supplier and the Acquirer’s designated representatives when retained within the government.

          - Concurrence of Minor (Class II) Changes

(1) The Supplier shall submit the Minor (Class II) change to the Configuration Approval Authority for concurrence, approval, or disapproval after the classification review and prior to delivery of the product.

      - Requests for Variance (RFV)

Principle CCM-9: If it is necessary to temporarily depart from approved product configuration information, a request for variance is identified, classified, documented, coordinated, evaluated, and dispositioned.

- The Supplier shall obtain the Acquirer’s or its designated representative approval on any Configuration Item or component, including COTS hardware or software, which is non-conforming to its requirements by preparation and submittal of an RFV.

NOTE: RFVs may be submitted by an Engineering Variance (i.e., before the item is produced and the Supplier is aware in advance of a need to deviate from the items requirements), or Production/Procurement Variance (i.e., a non-conformance is discovered after an item is produced).

- If the Acquirer determines that the Engineering Variance would be permanently acceptable, then the Supplier shall submit an ECP to accomplish the change and results in closure or removal from the CSA information.

- The Supplier shall provide for:

  - Production/Procurement Variance (aka Post Production RFV) – the quantity of items affected by the nonconformance and the extent the affected items do not meet the Acquirer's contractual requirements. The Supplier shall provide per unit and total cost impact and will be calculated based on estimates to complete repairs or replace the affected item(s). When supplies or services are accepted with minor nonconformances, the identified amount shall be used to determine if the savings to the Supplier exceed the cost to the Acquirer to modify the contract. When supplies or services are accepted with critical or major nonconformances, the Acquirer’s contracting officer must modify the contract to provide for an equitable price reduction or other consideration as per FAR 46.407

-- Nonconforming Supplies or Services.

  - Engineering Variance (aka Pre-Production RFV) – the quantity of items affected and the extent of the proposed departure from an approved product definition information. In case of self-inflicted need for departure from design and/or requirements, the Supplier shall provide per unit and total savings when submitting the request for Engineering Variance. When supplies or services are accepted with critical or major nonconformances, the Acquirer’s contracting officer must modify the contract to provide for an equitable price reduction or other consideration as per FAR 46.407 -- Nonconforming Supplies or Services.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

The Supplier shall document the following elements of the dispositioned RFV in the CSA system: Type of Variance (non-conformance)

  - Description; Impact of the variance (pre-production or post production) inclusive of any impact to any additional product/part;

  - Criteria used in establishing the need for corrective action appropriate to the effects of the variance and when applicable;

  - Action taken/accomplished by the Supplier to prevent reoccurrence of the non-conformance.

NOTE: The Acquiring Activity may cite DI-SESS-80640 Request for Variance, for specifying the delivery of the data product that emanates from meeting this requirement.

        - Recurring Variances

- The Supplier shall obtain the Acquirer’s approval before submitting a reoccurring RFV with the same nonconformance. Corrective action plans included in a RFV should reduce the need for recurring Variances.

- The Supplier shall evaluate recurring engineering variances for possible submittal of an ECP. RFVs are limited to their documented effectivity, which must be less than or equal to the remaining contract period for all produced items/units. RFVs can only apply to the contract under which it is submitted. The RFV cannot apply to future contracts.

        - Classification of Variances

- The Supplier shall use content in “DD Form 1694” for assigning a Critical/Major or Minor classification to an RFV. Classification based on the impact of the variance:

  - Critical. Use this classification when it is a departure from requirements affecting one or more of the following:

    - Safety

    - Human health

    - Environment

    - Security (local program or national)

  - Major. Use this classification when it is a departure from requirements affecting one or more of the following:

    - Performance or operational limits

    - Interchangeability, reliability, survivability, maintainability or durability of the item or its repair parts

    - Structural strength

    - Effective use or operation

    - Weight, moment of inertia, center of gravity

    - Appearance

    - Limits on product use or operation

    - Temporary use of alternate items

    - When the configuration documentation defining the requirements for the item classifies the departure from the requirement as major

  - Minor. Use this classification when it consists of a departure that does not involve any of the factors listed for critical or major, or when the configuration documentation defining the requirements for the item classifies the departure from the requirement as minor.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

NOTE: Departures from the requirements that do not meet the definition of critical or major and are not classified in any configuration documentation (i.e., unlisted characteristic) are treated as minor.

- The Supplier shall submit to the Acquirer for final decision, any classification disagreements between the Supplier and the Acquirer’s designated representatives.

- Suppliers shall provide an implementation timeline for Critical and Major classified RFVs.

        - RFV Decisions

The Acquirer is configuration approval authority for all Critical and Major RFVs. The Configuration Approval Authority for all Critical, Major or Minor RFVs is the Acquirer or Acquirer’s designated representative.

- The Supplier shall submit all proposed Critical and Major RFVs to the Acquirer with data from the Acquirer’s designated representative, if available.

- The Supplier shall submit all requested Minor RFVs to the Acquirer or Acquirer’s designated representative.

        - Minor RFVs

(1) The Supplier shall assign an RFV classification of Minor to an RFV that does not meet the criteria for a Critical or Major.

      - Specification Change Notice (SCN)

- The Supplier (design activity for, or the current change authority of a specification not generated by word processing) shall prepare a separate proposed SCN for each contractual specification that would require revision.

- The Supplier shall submit SCNs in conjunction with any ECP that affects a specification. Program/project organizations not being in possession of interactive digital files shall prepare a proposed change on specifications in NOR format.

NOTE: The Acquiring Activity may cite DI-SESS-80643 Specification Change Notice, for specifying the delivery of the data product that emanates from meeting this requirement.

      - Notices of Revision (NORs)

- The Supplier shall submit a NOR or contractor format if so allowed for each document affected by an ECP.

- The Supplier shall prepare and submit with each NOR all information required to disposition the NOR and its associated ECP.

NOTE: The Acquiring Activity may cite DI-SESS-80642 Notice of Revision, for specifying the delivery of the data product that emanates from meeting this requirement.

    - Configuration Status Accounting (CSA)

Principle CSA-1: Configuration Status Accounting (CSA) provides an accurate, timely information base concerning a product and its product configuration information throughout the product life cycle.

Principle CSA-2: Information about the product and the product configuration information are captured as CM tasks are performed; reporting is accessible to support program/project activities as needed.

CSA formalizes the recording and reporting of the accurate and timely established product configuration information. CSA provides status of proposed changes, the implementation of approved changes and changes occurring to product units due to operation and maintenance.

- The Supplier shall maintain the legitimacy of the CSA by storing valid data in the CSA system.

- The Supplier shall perform CSA analysis to detect and identify trends and report problems.

      - Supplier CSA System

The Supplier may utilize any information technology tools or systems desired to meet CSA functional process requirements described herein.

- The Supplier’s CSA system shall be closely linked to the data repository used to store and manage the product configuration and product operational information.

This close linkage may be accomplished via use of an all-encompassing product data management system (with CM, data repository, requirements management, and traceability functionality all in the same product) or by integration of separate stand-alone IT tools/systems.

- The Supplier shall use a CSA system to capture, store, and maintain product configuration information about the product that is either provided by the Acquirer or created by the Supplier during the duration of the contract.

- The Supplier shall establish and maintain a CSA system for support of the CDRL Data Items that has the following capabilities:

  - Capture and maintain the currently approved product configuration information by the identification number assigned to each Configuration Item and component.

  - Capture and maintain all historical product configuration information with traceability detailing product configuration from contract award to the present.

  - Store and manage all product configuration information.

  - Identify dependencies among Configuration Items.

  - Report the status of both proposed and approved configuration changes and variances to all Configuration Items.

  - Record, track and report implementation status and engineering release of configuration documentation for approved configuration changes.

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Record, track and report the status of corrective actions (repairs, “use as is”) for nonconforming materiel addressed by the RFVs, which affects the configuration of any Configuration Items.

  - Record and report the planning and results of configuration audits to include the status and final disposition of identified discrepancies.

  - Provide performance metrics that measure the execution of the Supplier’s CM process and functions.

  - Record and report all product data inherited from Sub-tier suppliers, to include vendor part numbers and serial numbers, and provide clear traceability from these components to their respective next higher assemblies.

- The Supplier shall:

  - Make CSA system and product configuration information readily available to the Acquirer or Acquirer’s designated representative, via on-site inspection, remote access, or regular submissions of CSA information and updates in accordance with the contract.

  - Comply with Cyber Security requirements for the purpose of interoperating with the Acquirer’s system as in an integrated digital environment.

  - Adequately secure, safeguard and retrieve, after extended storage, recorded product configuration information.

- The Supplier shall provide a digital delivery to the Acquirer upon the completion of the contract, CSA information and configuration baseline documentation created, collected, and managed during the contract. The form, format and delivery date for this data delivery shall be in accordance Contract Data Requirements List (CDRL) specified within the contract.

NOTE: The Acquiring Activity may cite DI-SESS-81253 Configuration Status Accounting Information, for specifying the delivery of the data product that emanates from meeting this requirement.

- The Supplier shall document the incorporation of all retrofit changes to those units identified for retrofit when so tasked by contract provisions.

NOTE: The Acquiring Activity may cite DI-SESS-81245 Installation Completion Notification, for specifying the delivery of the data product that emanates from meeting this requirement.

      - CSA Analysis Requirements

Principle CSA-3: Metrics derived from configuration status accounting information are used to evaluate and improve CM process effectiveness.

(1) The Supplier shall evaluate CSA corrective actions to verify that problems have been resolved, metrics of adverse trends have been reversed and changes have been correctly implemented in the appropriate processes and products; and determine whether additional problems have been introduced.

Suggested performance metrics may include but are not limited to those that are listed in 3.1.1.

    - Configuration Verification and Audits

Principle CVA-1: Verify CM processes to ensure appropriate consistency is established and maintained in their execution.

Principle CVA-2: Verifying a product’s compliance with the physical, functional, and interface requirements in approved product configuration information confirms the basis for managing product configuration.

Principle CVA-3: Verify the implementation of each change to ensure consistency is maintained between the product, its configuration information, and related support assets.

Principle CVA-4: Configuration audits are a summation of the configuration verification process, where necessary to establish approved configurations at key points in the product life cycle.

The audit verifies that the product during development or modification has achieved specified requirements and the design is accurately and completely documented in configuration baseline information. Verifying the documentation determines that it is adequate for its intended purposes and accurately reflects a design compliant with the product’s functional and physical requirements. DoD utilizes Functional and Physical Configuration Audits (FCA/PCA) to achieve these goals.

      - Common Supplier Audit Responsibilities

- The Supplier shall:

  - Conduct the FCAs and PCAs for designated Configuration Items to verify the system allocated and product configuration baselines including the prime supplier’s designated Sub-tier level.

  - Support and participate in the FCAs and PCAs for related Configuration Items, as identified by the Acquirer.

  - Conduct incremental, lower-level Configuration Item audits leading up to the system level audits.

  - Establish the schedule for audits to be conducted at Supplier or Sub-supplier facilities including the agenda for each configuration audit in consonance with the Integrated Master Schedule (IMS) and obtain Acquirer’s approval.

--`,`,,,```,````,```

Confirm that each configuration audit schedule is compatible with the availability of the necessary information and contract articles (e.g., system engineering data, released engineering documentation, trade study results, producibility analysis results, risk analysis results, product specifications, quality control records, manuals, drawings, reports, hardware, Interface Design Document (IDD), and SVD for software).

  - Sub-suppliers participate in planning and conducting configuration audits and prepare an audit plan to identify their requirements and responsibilities.

  - Designate a representative for the audit.

  - Provide a current listing of all RFVs, SCNs, NORs, and ECPs against the Configuration Item either requested or approved.

  - Participating Supplier and Sub-supplier personnel are prepared to discuss the technical detail of the presented materiel.

  - Participate in the resolution of discrepancies identified during the FCA/PCA.

  - Summarize conclusions and add appropriate comments summarizing the side meetings and the main meeting into the official minutes.

  - Document significant questions and answers, action items, conclusions, and recommended courses of action resulting from each audit.

  - Record all discrepancies identified during the audit and process each one until it is closed out or a suitable residual task, including identification of responsible activities and suspenses, have been established and which will lead to the close out of the discrepancy/action item.

  - Verify the implementation of engineering changes and variance corrective actions to confirm consistency is maintained between the product and its product configuration information.

- When audits are conducted in increments, the Supplier shall support a final audit to verify that all requirements of the FBL, ABL, and PBL have been satisfied.

- Supplier shall complete all audit entry and exit criteria prior to completion or closeout of the audit.

NOTE: The Acquirer should consult IEEE 15288.2 Technical Reviews and Audits and all associated Addenda and Amplifications for tailoring PCA Entry and Exit Criteria, which are determined through acceptance of the Supplier's Systems Engineering Management Plan (SEMP).

NOTE: The Acquiring Activity may cite DI-SESS-81646 Configuration Audit Plan, DI-ADMN-81249 Conference Agenda, and DI-ADMN-81250 Conference Minutes, for specifying the delivery of the data product that emanates from meeting this requirement.

      - Functional Configuration Audit (FCA)

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

The objective of the FCA is to verify the CI’s and system’s performance against its approved functional and allocated configuration documentation, to ensure the Configuration Item’s and system’s specifications and interfaces are accurately and adequately defined, and to ensure the Supplier’s change control system is adequate to control the processing and release of configuration documentation. The Computer Software Configuration Item (or CSCI) FCA ensures that all requirements have a related test step, and the test, as performed by test engineers, passes successfully.

- When FCAs are conducted in increments, the Supplier shall conduct a final system-level FCA to confirm that the system has been satisfied.

- The Supplier shall conduct the FCA to verify that the Configuration Item meets its performance requirements as documented in the approved FCD, ACD and is consistent with the up-to-date PCD.

- Each FCA non-compliance shall be documented and provided to the Acquirer for appropriate action.

- The Supplier shall use test data from the verification of the Configuration Item for the FCA.

- The Supplier shall audit the Configuration Item with prior Acquirer approval of the FBL and ABL of the Configuration Item or system involved.

        - Supplier FCA Responsibilities

- The Supplier shall provide the following information to the Acquirer prior to the audit date:

  - List of items to be audited to include documents.

  - List of tasks to be accomplished at the FCA for the Configuration Item.

  - Matrix that identifies and provides traceability for all requirements; includes a cross reference to the test plans, test procedures, test programs with automatic/automated test equipment (when applicable), test reports (results of demonstrations, inspections and analyses for each requirement), and any known deficiencies supported by applicable deficiency report numbers.

- The Supplier shall provide the following information and data necessary to support the FCA.

  - The configuration documentation for the Configuration Item being audited.

  - All approved ECPs.

  - An account of the ECPs incorporated and tested as well as proposed.

  - Test plans, specifications, descriptions, procedures, recorded results, and analyses for the Configuration Item.

  - A general discussion of the entire Configuration Item test effort delineating problem area accomplishments.

  - Configuration Item requirements that were not met, including a proposed remedy for each item.

  - Complete list of test requirements not yet performed and a plan for accomplishing each requirement.

  - Analyses or simulations for those requirements, which cannot be completely verified through the use of testing.

  - Provide the following for Computer Software Configuration Items:

    - Database characteristics, storage allocation data, and timing and sequencing characteristics for compliance with specified requirements.

    - Documents that comprise or describe the contents or the use of the software product for format and completeness (e.g., Software Product specification (SPS), User’s Manual, SVD))

    - Records that reflect the changes made to the developmental configuration for the Computer Software Configuration Items.

    - Listing of all versions of the developmental and non-developmental software for the Computer Software Configuration Items that are in the software library.

    - Findings of all internal CM and software QA audits of the Computer Software Configuration Items.

  - Preliminary Design Review (PDR) and Critical Design Review (CDR) minutes. The Acquirer examines to ensure that all findings have been incorporated and completed and that all action items are closed.

- The Supplier shall provide the FCA minutes to include the requirements traceability and test matrix.

      - Physical Configuration Audit (PCA)

The purpose of the PCA is to provide assurance to the Acquirer that the product configuration documentation (including CAD models, drawings and software source code) accurately reflects the validated product, and the documentation is a clear, complete and accurate representation of the intended design and to establish the final product baseline. The Supplier and Acquirer conducts a PCA to examine the as-built configuration of a validated CI, comparing results against its design documentation plus any approved changes after a successful CDR. The software PCA is conducted after producing a software build, the build has to be produced and checked that it can be installed on the device or computer during a PCA and verified against the SVD, and is witnessed by QA or any other entity established by the Acquiring activity. This would include verifying all software updates have been applied, as listed on the problem reports for this build. Each PCA non-compliance identified during the audit is documented by CM and evaluated by the project lead for appropriate action.

- The Supplier shall conduct the PCA to verify that the PCD is consistent with the validated Configuration Item.

- The Supplier shall confirm compatibility between the Configuration Item with the Program Parts Selection List (PPSL) and examine the Configuration Item to ensure that the proper parts are actually installed.

- When PCAs are conducted in increments, the Supplier shall conduct a final system-level PCA to confirm the system PBL is correct.

- The Supplier shall document each PCA non-compliance and provided to the Acquirer for appropriate action.

- If any portions of the inspections, tests, or audits need to be re-accomplished, the Supplier shall invite the Acquirer or Acquirers designated representative to witness the required inspections, tests, and audits.

        - Supplier PCA Responsibilities

- The Supplier shall provide the following information to the Acquirer prior to the audit date:

  - Data describing the item configuration, to include:

    - Configuration Item specifications.

    - A list delineating both approved and outstanding changes against the CI.

    - Complete shortage list.

    - Acceptance test procedures and associated test data from those procedures from the successful FCA.

    - Engineering drawing index including revision letters.

    - Operating and support manuals; including operator’s manuals, maintenance manuals, illustrated part breakdown, programmer’s manuals, diagnostic manuals, etc.

    - Proposed Material Inspection and Receiving Report, DD Form 250.

    - Approved nomenclature and nameplates.

    - SVDs for software.

    - FCA official minutes for each Configuration Item unless being performed incrementally.

    - Status of the current Original Equipment Manufacturer (OEM)/Supplier Quality Assurance Programs.

    - Parts Selection List.

    - Interface Design Document for software.

    - Current approved issue of hardware development and software and interface requirements specifications to include approved SCNs and approved variances.

    - Operational test plans, specifications, descriptions, procedures, recorded results, and analyses for the CI.

    - Configuration Item requirements that were not met, including a proposed remedy for each CI.

    - Complete list of operational test requirements not yet performed and a plan for accomplishing each requirement.

    - Analyses or simulations for those requirements, which cannot be completely validated through the use of testing.

    - Identification of all changes made during test.

    - Identification of any required changes not completed.

    - All configuration documentation, or electronic representations of the same, required to identify the CI.

  - Identification of any difference between the physical configurations of the selected production CI(s) and the development CI(s) used for the FCA, including certifications or demonstrations that these differences do not degrade the functional characteristics of the selected CI(s).

  - Form, fit, and function of the product model data as well as interface or integration data as proof that the product model and derived output products are current. First Article Tests/product verification could be used to provide information in support of PCA after CDR.

- The Supplier shall provide the following information and data necessary to support the PCA and provide within the minutes:

  - Complete product drawings/associated lists package (3D model type, cut-away drawings, and/or Model Based Definitions) and associated manufacturing instruction sheets (and/or Computer Aided Manufacturing (CAM) data) for each item of hardware or software identified.

  - Completed inspection reports for selected drawings (3D model type or cut-away drawings and/or Model Based Definitions) and associated manufacturing instructions (and/or CAM data).

--`,`,,,```,````,```,``,,,,,,`,`-``,,`,,,,-``,,`,,,,---

Records of the configuration baselines to compare with the Supplier’s engineering change control procedures and release system, including interim releases of spares/repair parts provisioned prior to PCA to confirm delivery of currently configured spares/repair parts.

  - Confirmation on the completion of the inspection and test of Sub-supplier equipment end items at point of manufacture.

  - PPSL to support examination of the Configuration Item to verify that the proper parts are actually installed.

      - Post-Audit Action

(1) The Supplier shall complete post-audit actions as assigned/directed.

      - Configuration Audit Summary

- The Supplier shall:

  - Prepare Configuration Audit Summary Report consisting of the applicable audit action items and discrepancy information that was derived from the applicable audit checklist package.

  - Prepare a detailed configuration audit summary report after successful verification of the completed action items has been conducted.

  - Provide the completed configuration audit summary report.

NOTE:	The Acquiring Activity may cite DI-SESS-81022 Configuration Audit Summary Report, for specifying the delivery of the data product that emanates from meeting this requirement.